# Onyx — Changes applied to Circle

Onyx · multi-process OS on Raspberry Pi 4

Onyx uses Circle (rsta2/circle) as its hardware-abstraction + driver layer, kept as close to upstream as possible. The handful of changes we need live in a fork, pulled in as a git submodule:

- Submodule: circle/ → https://github.com/stephaneweg/circle.git (.gitmodules), branch onyx.

- Upstream base: https://github.com/rsta2/circle.git, tag Step51.1.1 (remote upstream) — Step51 until 2026-10-09, when upstream's 51.1 and 51.1.1 were merged in (see Upstream merges below).

- The superproject pins a specific fork commit; git submodule status shows it (currently Step51-6-g…).

The patches on top of upstream are small and surgical. The diffs below are the output of git diff Step51..onyx inside circle/ at the time each was written (since the merge of 51.1.1, compare with Step51.1.1). To reproduce:

git -C circle remote -v                  # origin = the fork, upstream = rsta2/circle
git -C circle log  --oneline --no-merges Step51.1.1..onyx
git -C circle diff Step51.1.1..onyx

Design rule. Patch Circle as little as possible. When a change can be header-only (no library rebuild), it is. Anything that can live in our own kernel instead (for example the cooked-mouse changed-button handling, done in kernel/'s mouse stub) is not a Circle patch.

Some in-source comments still say "Zircon" (the project's former codename) instead of "Onyx" — cosmetic only.

## Summary

| # | Patch | Files touched | Rebuild |
| --- | --- | --- | --- |
| 1 | Keyboard map decoupled from the kernel (no compiled-in country maps; loaded at runtime) · MAX_TASKS 20 → 40 | input/keymap.{h,cpp} (+ 7 keymap_*.h deleted), usb/usbkeyboard.h, input/keyboardbehaviour.h, sysconfig.h | libinput (+ kernel for MAX_TASKS) |
| 2 | Heap large-block reuse (fix per-launch canvas leak) + free-list byte accounting | heapallocator.{h,cpp}, sysconfig.h | libcircle |
| 3 | Free-space accounting in the page allocator + memory accessors (for the meminfo kapi) | memory.h, pageallocator.{h,cpp} | libcircle (memory.h header-only) |
| 4 | Multi-core enabled (ARM_ALLOW_MULTI_CORE): core 1 runs the sound producer | sysconfig.h | every library (clean rebuild) + kernel |
| 5 | Shift + navigation keys (KeyShiftUp… appended to TSpecialKey, xterm ;2 sequences) | input/keymap.{h,cpp} | libinput, libusb + the .kmap files |
| 6 | Partial display update C2DGraphics::UpdateDisplay (x, y, w, h) (the compositor's dirty rectangles) | 2dgraphics.{h,cpp} | libcircle |
| 7 | Yielding SD waits: weak hooks for the FatFs volume lock and the EMMC wait loop | addon/fatfs/ffsystem.cpp, addon/SDCard/emmc.cpp | libfatfs, libsdcard |
| 8 | Sector cache in the FatFs disk layer (write-through), one bounce buffer per volume | addon/fatfs/diskio.{h,cpp} | libfatfs |
| 10 | Multi-cluster transfers in f_read / f_write (one SD command across contiguous clusters) | addon/fatfs/ff.c | libfatfs |
| 9 | High Speed SD at run time (CEMMCDevice::SetHighSpeed) + the host's High Speed Enable bit | addon/SDCard/emmc.{h,cpp} | libsdcard |
| 11 | FatFs fast seek on (FF_USE_FASTSEEK 1: kapi_seek in a GameCube disc image) | addon/fatfs/ffconf.h | libfatfs + the kernel (the FIL layout: clean rebuild) |
| 14 | DHCP restart (joining another Wi-Fi network while running; the reconfiguration itself: the kernel's link-time wrap, no hostap change) | include/circle/net/dhcpclient.h, lib/net/dhcpclient.cpp | libnet (+ the kernel) |
| 13 | The SD card's partitions as volumes SD: SD1: … SD3: + exFAT on | addon/fatfs/ffconf.h, addon/fatfs/diskio.cpp, addon/fatfs/ff.c | libfatfs + the kernel (FSIZE_t is 64-bit: clean rebuild) |
| 17 | The host name set after the constructor (CNetSubSystem::SetHostname: system.ini's hostname=) | include/circle/net/netsubsystem.h | none (inline) — the kernel |
| 7b | Yielding SD waits, many short ones: a second weak hook OnyxDriverPoll at each turn of a short wait | addon/SDCard/emmc.cpp | libsdcard |
| 20 | TCP: real duplicate ACKs only (RFC 5681 §2) — the peer's data segments no longer start a fast retransmit | lib/net/tcpconnection.cpp | libnet |
| 21 | TCP: RTO 1 s initial (was 3 s; the 1 s minimum kept — 200 ms was tried and undone), Karn after a fast retransmit | lib/net/retranstimeoutcalc.cpp, include/circle/net/retranstimeoutcalc.h, lib/net/tcpconnection.cpp | libnet |
| 22 | CSocket::Send's count: the bytes queued when a later chunk times out | lib/net/socket.cpp | libnet |
| 23 | Wi-Fi: the firmware's wsec from the IE's ciphers (group + pairwise: TKIP 2, AES 4, as Linux's brcmfmac) -- WPA2 was always "aes": a WPA/WPA2 mixed-mode network (pairwise CCMP, group TKIP, common on 2.4 GHz) associated but no broadcast was decoded (no DHCP offer: the link stayed down) | addon/wlan/ether4330.c (iewsec, setauth) | libwlan |
| 25 | TCP: a closed connection's retransmission timer stopped, a late one ignored — it was a kernel panic (Unexpected state 0); a reset wakes a waiting sender | lib/net/tcpconnection.cpp | libnet |
| 26 | Wi-Fi: the chip polled when the network has its own core (the SDIO card interrupt came 2.5–5 ms late), a scan over both bands with the configured networks probed by name, 5 GHz preferred, a frame in one SDIO command, the frames' locks without a yield, the SDIO bus at 50 MHz, no A-MPDU for the frames sent (half of them were lost during a download) | addon/wlan/ether4330.c, addon/wlan/emmc.c, addon/wlan/p9util.cpp | libwlan |
| 27 | TCP: window scaling (RFC 7323), a receive window that follows the receive queue (it was a constant), a 256 KB transmit threshold, delayed acknowledgements (one for eight segments) | lib/net/tcpconnection.cpp, include/circle/net/tcpconnection.h, include/circle/net/sizes.h | libnet |
| 28 | USB volumes: f_mkfs and the labels on (FF_USE_MKFS, FF_USE_LABEL), FatFs' objects checked again once the volume lock is held, the volume mutex kept across an unmount, a yield between two USB transfers, SCSI SYNCHRONIZE CACHE for the eject | addon/fatfs/ffconf.h, ff.c, ffsystem.cpp, diskio.cpp, lib/usb/usbmassdevice.cpp, include/circle/usb/usbmassdevice.h | libfatfs, libusb + the kernel (no structure changes: no clean rebuild) |
| 29 | The Pi 5's high RAM registered once: seg0 = [1 GB, 8 GB) is all of it; no reclaim above it (its fallback registered [4 GB, 8 GB) a second time: two allocators, the same frames); the crash record at seg0's top. The Pi 4 unchanged | lib/memory64.cpp | libcircle (the Pi 5's, circle5/) |
| 12 | 2D DMA with a source stride (a rectangle read in place: no gathering) + an asynchronous partial update (the compositor yields instead of spinning) | dmachannel.{h,cpp}, dma4channel.{h,cpp}, bcmframebuffer.{h,cpp}, 2dgraphics.{h,cpp} | libcircle |

## 1. Keyboard map decoupled from the kernel (+ MAX_TASKS)

Why. Circle's cooked-mode keyboard bakes the country maps into the binary (one selected at boot via keymap=) and offers no way to change it afterwards. Onyx wants live layout switching and layouts that ship as data files (SD:/etc/keymaps/*.kmap), so a new layout needs no kernel rebuild — and the kernel carries no keyboard tables at all.

What.

- Expose the keyboard's CKeyMap up the device stack (keyboardbehaviour.h, usbkeyboard.h gain a GetKeyMap()), so the kernel can reach the live map and fill it through the existing public CKeyMap::SetEntry / ClearTable.

- Drop the compiled-in maps. keymap.cpp no longer #includes the keymap_*.h tables, and CKeyMap loses s_DefaultMap[], s_MapDirectory[] and LookupDefaultMap(); the constructor just zeroes the map (every entry KeyNone). The 7 stock tables (keymap_{de,dv,es,fr,it,uk,us}.h, 905 lines) are deleted from the Circle tree — they now live in the Onyx repo at tools/keymaps/maps/ and are compiled to .kmap blobs by tools/keymaps/genkeymaps.py.

The kernel fills the live keyboard from a .kmap file via SetEntry (kapi_set_keymap_data, ABI v27), re-applying it whenever a keyboard (re-)attaches. MAX_TASKS goes 20 → 40: the network stack adds long-lived background tasks (net / DHCP / WPA supplicant + NTP + IRC). (Onyx's own scheduler has since dropped the fixed table: its tasks are a linked list with no limit, docs/02 §5, so MAX_TASKS no longer bounds Onyx; the patch stays for Circle's own code.)

--- a/include/circle/input/keymap.h
+++ b/include/circle/input/keymap.h
@@ -130,8 +130,9 @@ public:
 
    u8 GetLEDStatus (void) const;
 
-private:
-   static const void *LookupDefaultMap (const char *pLocale);
+   // Zircon: the keyboard map is empty until the kernel fills it at runtime via
+   // ClearTable/SetEntry from a SD:/etc/keymaps/*.kmap file -- no country map is
+   // compiled in (see CKeyMap::CKeyMap and kapi_set_keymap_data).
 
 private:
    u16 m_KeyMap[PHY_MAX_CODE+1][K_CTRLTAB+1];
@@ -141,8 +142,6 @@ private:
    boolean m_bScrollLock;
    
    static const char *s_KeyStrings[KeyMaxCode-KeySpace];
-   static const u16 s_DefaultMap[][PHY_MAX_CODE+1][K_CTRLTAB+1];
-   static const char *s_MapDirectory[];
 };

--- a/lib/input/keymap.cpp
+++ b/lib/input/keymap.cpp
@@ -96,60 +96,16 @@ const char *CKeyMap::s_KeyStrings[KeyMaxCode-KeySpace] =
    "."         // KeyKP_Period
 };
 
-#define C(chr)     ((u16) (u8) (chr))
-
-const u16 CKeyMap::s_DefaultMap[][PHY_MAX_CODE+1][K_CTRLTAB+1] =
-{
-   {
-       #include "keymap_de.h"
-   }, {
-       #include "keymap_dv.h"
-   }, {
-       ... (fr, es, it, uk, us)
-   }
-};
-
-const char *CKeyMap::s_MapDirectory[] = { "DE","DV","ES","FR","IT","UK","US", 0 };
-
+// Zircon: the kernel compiles in NO country maps -- keyboard layouts ship as
+// SD:/etc/keymaps/*.kmap data files and are loaded at runtime through ClearTable/SetEntry
+// (see kapi_set_keymap_data). So a fresh keymap starts empty (every entry KeyNone).
 CKeyMap::CKeyMap (void)
 :  m_bCapsLock (FALSE), m_bNumLock (FALSE), m_bScrollLock (FALSE)
 {
-   const char *pLocale = CKernelOptions::Get ()->GetKeyMap ();
-   const void *pDefaultMap = LookupDefaultMap (pLocale);
-   ... (fallback to DEFAULT_KEYMAP)
-   memcpy (m_KeyMap, pDefaultMap, sizeof m_KeyMap);
+   memset (m_KeyMap, 0, sizeof m_KeyMap);      // KeyNone == 0
 }
@@ -336,17 +292,3 @@ u8 CKeyMap::GetLEDStatus (void) const
-const void *CKeyMap::LookupDefaultMap (const char *pLocale)
-{
-   for (unsigned nMap = 0; s_MapDirectory[nMap] != 0; nMap++)
-       if (strcmp (s_MapDirectory[nMap], pLocale) == 0)
-           return s_DefaultMap[nMap];
-   return 0;
-}

--- a/include/circle/input/keyboardbehaviour.h
+++ b/include/circle/input/keyboardbehaviour.h
@@ -69,6 +69,8 @@ public:
    u8 GetLEDStatus (void) const;
 
+   CKeyMap *GetKeyMap (void)   { return &m_KeyMap; }   // Zircon: runtime layout switch
+
 private:
    void GenerateKeyEvent (u8 ucKeyCode);

--- a/include/circle/usb/usbkeyboard.h
+++ b/include/circle/usb/usbkeyboard.h
@@ -62,6 +62,9 @@ public:
    boolean SetLEDs (u8 ucStatus);
 
+   // Zircon: access the cooked-mode key map so the layout can be switched at runtime.
+   CKeyMap *GetKeyMap (void)   { return m_Behaviour.GetKeyMap (); }
+
 private:
    void ReportHandler (const u8 *pReport, unsigned nReportSize);

--- a/include/circle/sysconfig.h
+++ b/include/circle/sysconfig.h
@@ -273,7 +277,8 @@
 #ifndef MAX_TASKS
-#define MAX_TASKS      20
+#define MAX_TASKS      40      // Onyx: bumped from 20 -- net stack adds
+                       // background tasks (net/DHCP/WPA + NTP/IRC)
 #endif

(The 7 lib/input/keymap_*.h tables are deleted — 905 lines — and not shown here.)

## 2. Heap large-block reuse (fix per-launch leak)

Why. Circle's CHeapAllocator keeps freed blocks on per-size bucket free lists, but the buckets top out at 0x80000 = 512 KB. A block bigger than the top bucket cannot be returned to any free list and is lost on Free(). Onyx allocates large blocks routinely — a window canvas is up to ~3 MB (1024×768×4) — so every app launch leaked its canvas.

Fix. Add power-of-two "large" free lists alongside the buckets: a too-big block is rounded to its power-of-two class and pushed onto m_pLargeFreeList[exp] on free, and reused from it on allocate. A running m_nFreeListBytes counter (exposed via GetFreeListSpace()) feeds the meminfo accounting (patch 3). Rebuild libcircle after this change.

--- a/include/circle/heapallocator.h
+++ b/include/circle/heapallocator.h
@@ -40,6 +40,12 @@ ASSERT_STATIC (DATA_CACHE_LINE_LENGTH_MAX >= 16);
 #define HEAP_BLOCK_MAX_BUCKETS 20
 
+// Onyx: blocks larger than the biggest bucket are rounded up to a power of two and
+// kept on a per-power free list (index = log2 of the rounded size), so large blocks
+// (window canvases, big file buffers, ...) are reused instead of lost on free. 32
+// classes cover any realistic size.
+#define HEAP_LARGE_LISTS   32
+
 struct THeapBlockHeader
@@ -83,6 +89,10 @@ public:
    size_t GetFreeSpace (void) const;
 
+   /// \return Onyx: bytes of freed blocks currently on the bucket/large free lists
+   ///     (reusable). Add to GetFreeSpace() for the true total free space.
+   size_t GetFreeListSpace (void) const    { return m_nFreeListBytes; }
+
    /// \param nSize Block size to be allocated
@@ -95,8 +105,8 @@ public:
    /// \param pBlock Memory block to be freed
-   /// \note Memory space of blocks, which are bigger than the largest bucket size,
-   ///   cannot be returned to a free list and is lost.
+   /// \note Onyx: blocks bigger than the largest bucket are rounded to a power of two
+   ///   and returned to a per-power free list (reused), so they are NOT lost.
    void Free (void *pBlock);
@@ -117,6 +127,8 @@ private:
    THeapBlockBucket m_Bucket[HEAP_BLOCK_MAX_BUCKETS+1];
+   THeapBlockHeader *m_pLargeFreeList[HEAP_LARGE_LISTS];   // per-power-of-2 (Onyx)
+   size_t       m_nFreeListBytes;  // Onyx: bytes on the bucket+large free lists
    CSpinLock    m_SpinLock;

--- a/lib/heapallocator.cpp
+++ b/lib/heapallocator.cpp
@@ -53,6 +53,45 @@ void CHeapAllocator::Setup (uintptr nBase, size_t nSize, size_t nReserve)
    m_nReserve = nReserve;
+
+   for (unsigned i = 0; i < HEAP_LARGE_LISTS; i++)     // Onyx: large-block free lists
+       m_pLargeFreeList[i] = 0;
+   m_nFreeListBytes = 0;
+}
+
+// Onyx: round a too-big request up to a power-of-two class; return its free-list index.
+static int HeapLargeClass (size_t *pnSize)
+{
+   size_t nRounded = HEAP_BLOCK_ALIGN; int nExp = 0;
+   while (nRounded < *pnSize) { nRounded <<= 1; if (++nExp >= HEAP_LARGE_LISTS) return -1; }
+   *pnSize = nRounded; return nExp;
+}
+// Free-list index of an already-rounded large block (stored size = ALIGN << exp).
+static int HeapLargeIndex (size_t nRoundedSize)
+{
+   int nExp = 0;
+   while (((size_t) HEAP_BLOCK_ALIGN << nExp) < nRoundedSize)
+       if (++nExp >= HEAP_LARGE_LISTS) return -1;
+   return nExp;
 }
@@ -96,12 +135,28 @@ void *CHeapAllocator::DoAllocate (size_t nSize)
+   // Onyx: too big for every bucket -> round to a power-of-two class so it can be
+   // reused from m_pLargeFreeList instead of being lost on free.
+   int nLargeExp = -1;
+   if (pBucket->nSize == 0) nLargeExp = HeapLargeClass (&nSize);
+
    THeapBlockHeader *pBlockHeader;
    if (pBucket->nSize > 0 && (pBlockHeader = pBucket->pFreeList) != 0) {
        pBucket->pFreeList = pBlockHeader->pNext;
+       m_nFreeListBytes -= pBlockHeader->nSize;    // Onyx
+   } else if (nLargeExp >= 0 && (pBlockHeader = m_pLargeFreeList[nLargeExp]) != 0) {
+       m_pLargeFreeList[nLargeExp] = pBlockHeader->pNext;
+       m_nFreeListBytes -= pBlockHeader->nSize;    // Onyx
    } else { ... bump m_pNext ... }
@@ -226,6 +281,7 @@ void CHeapAllocator::DoFree (void *pBlock)   // (bucket free path)
            pBucket->pFreeList = pBlockHeader;
+           m_nFreeListBytes += pBlockHeader->nSize;    // Onyx
@@ -237,6 +293,19 @@ void CHeapAllocator::DoFree (void *pBlock)
+   // Onyx: no matching bucket -> power-of-two "large" block: return it to its size
+   // class free list so it is reused on the next same-class allocation (no leak).
+   int nLargeExp = HeapLargeIndex (pBlockHeader->nSize);
+   if (nLargeExp >= 0) {
+       m_SpinLock.Acquire ();
+       pBlockHeader->pNext = m_pLargeFreeList[nLargeExp];
+       m_pLargeFreeList[nLargeExp] = pBlockHeader;
+       m_nFreeListBytes += pBlockHeader->nSize;
+       m_SpinLock.Release ();
+       return;
+   }

--- a/include/circle/sysconfig.h
+++ b/include/circle/sysconfig.h
@@ -85,6 +85,10 @@
 #ifndef HEAP_BLOCK_BUCKET_SIZES
+// Small fixed buckets for common sizes. Larger requests (e.g. window canvases) no
+// longer need a matching bucket: Onyx's heap allocator rounds anything above the top
+// bucket up to a power-of-two class and reuses it from a per-power free list (see
+// HEAP_LARGE_LISTS / heapallocator.cpp), so they are no longer lost on free.
 #define HEAP_BLOCK_BUCKET_SIZES    0x40,0x400,0x1000,0x4000,0x10000,0x40000,0x80000
 #endif

## 3. Free-space accounting (for the meminfo kapi)

Why. The meminfo kapi (and the memory monitor) reports total / free / app memory. The "free" figure must include both the unallocated region and the freed blocks/pages sitting on the allocators' free lists (reusable). Circle exposed only the unallocated region, so we add free-list accessors. memory.h is header-only; the pageallocator counter needs a libcircle rebuild.

--- a/include/circle/memory.h
+++ b/include/circle/memory.h
@@ -155,6 +155,25 @@ public:
    static void PageFree (void *pPage)  { s_pThis->m_Pager.Free (pPage); }
 
+   // Onyx: free space (bytes) of the page allocator region not yet handed out.
+   static size_t GetPagerFreeSpace (void)  { return s_pThis->m_Pager.GetFreeSpace (); }
+   // Onyx: + freed pages on the pager free list (reusable).
+   static size_t GetPagerFreeListSpace (void) { return s_pThis->m_Pager.GetFreeListSpace (); }
+
+   // Onyx: freed heap blocks on the bucket/large free lists (reusable). Add to
+   // GetHeapFreeSpace(HEAP_ANY) for the true free heap.
+   size_t GetHeapFreeListSpace (void) const
+   {
+#if RASPPI >= 4
+       return s_pThis->m_HeapLow.GetFreeListSpace () + s_pThis->m_HeapHigh.GetFreeListSpace ();
+#else
+       return s_pThis->m_HeapLow.GetFreeListSpace ();
+#endif
+   }
+
    static void DumpStatus (void)

--- a/include/circle/pageallocator.h
+++ b/include/circle/pageallocator.h
@@ -48,6 +48,10 @@ public:
    size_t GetFreeSpace (void) const;
 
+   /// \return Onyx: bytes of freed pages currently on the free list (reusable). Add
+   ///     to GetFreeSpace() for the true total free space.
+   size_t GetFreeListSpace (void) const;
+
    void *Allocate (void);
@@ -67,6 +71,7 @@ private:
    TFreePage   *m_pFreeList;
+   unsigned     m_nFreeListCount;  // Onyx: pages currently on m_pFreeList
    CSpinLock    m_SpinLock;

--- a/lib/pageallocator.cpp
+++ b/lib/pageallocator.cpp
@@ -30,7 +30,8 @@ CPageAllocator::CPageAllocator (void)
-   m_pFreeList (0)
+   m_pFreeList (0),
+   m_nFreeListCount (0)
 {
 }
@@ -49,6 +50,11 @@ size_t CPageAllocator::GetFreeSpace (void) const
+size_t CPageAllocator::GetFreeListSpace (void) const   // Onyx: reusable freed pages
+{
+   return (size_t) m_nFreeListCount * PAGE_SIZE;
+}
@@ -68,6 +74,7 @@ void *CPageAllocator::Allocate (void)
        m_pFreeList = pFreePage->pNext;
+       m_nFreeListCount--;         // Onyx: page taken off the free list
@@ -103,6 +110,7 @@ void CPageAllocator::Free (void *pPage)
    m_pFreeList = pFreePage;
+   m_nFreeListCount++;             // Onyx: page returned to the free list

## 4. Multi-core enabled (ARM_ALLOW_MULTI_CORE)

include/circle/sysconfig.h: the define is un-commented, so Circle builds its multi-core support (CMultiCoreSupport, real spin locks, per-core stacks and MMU setup, IPIs). The Onyx kernel starts cores 1–3 at boot; core 1 is the sound producer (it renders the audio chunks ahead, see Kernel Internals §12); the scheduler, the processes and the interrupts stay on core 0. The memory map is unchanged (Circle always reserves the stacks of the 4 cores).

-//#define ARM_ALLOW_MULTI_CORE
+#define ARM_ALLOW_MULTI_CORE           // Onyx: core 1 runs the sound producer (kernel sys/sound.cpp)

Changing this define changes code in every Circle library: after updating the submodule, run make clean in each library before rebuilding them (the .o files do not depend on sysconfig.h) — see the Developer Guide §2.

## 5. Shift + navigation keys

Circle's keymap only has Ctrl variants of the navigation keys (KeyCtrlUp … → ESC[1;5A …); Shift + an arrow produced nothing. The patch adds eight special keys at the end of TSpecialKey (just before KeyMaxCode), so every existing code — the values stored in the .kmap layout files — keeps its number:

| Key | String |
| --- | --- |
| KeyShiftHome / KeyShiftEnd | ESC[1;2H / ESC[1;2F |
| KeyShiftPageUp / KeyShiftPageDown | ESC[5;2~ / ESC[6;2~ |
| KeyShiftUp / Down / Left / Right | ESC[1;2A / B / D / C |

The Onyx layouts (tools/keymaps/maps/*.h → genkeymaps.py → SD:/etc/keymaps/*.kmap) put them in the Shift column of the physical keys 0x4A–0x52. The kernel's key parser (gui/window.cpp, NextKey) turns any ESC[n;mX into the plain KEY_* code; apps read the modifier with kapi_get_modifiers (text selection in uikit::Textarea / RichTextBox).

## 6. Partial display update

C2DGraphics::UpdateDisplay () sends the whole off-screen buffer to the frame buffer. The Onyx compositor redraws only the damaged rectangles of the screen, so the patch adds an overload that sends one rectangle:

void UpdateDisplay (unsigned nPosX, unsigned nPosY, unsigned nWidth, unsigned nHeight);

It clips the rectangle to the screen, copies its rows into one contiguous block (a screen-sized buffer allocated on first use: cached memory, fast) and hands it to CBcmFrameBuffer::SetArea — the frame buffer's own 2D DMA copy with the destination pitch, as the full update uses. With VSync (page flipping) or a display that is not the frame buffer it falls back to the full UpdateDisplay ().

## 7. Yielding SD waits (FatFs lock + EMMC wait hooks)

The EMMC driver waits for the card in a busy loop (CEMMCDevice::TimeoutWait) unless NO_BUSY_WAIT is set, and Onyx's kernel is not preempted: every SD command kept the CPU, and a directory walk or a big read froze the GUI and the network 100–200 ms at a time. NO_BUSY_WAIT is global (USB, sound, every CGenericLock), so the patch adds two weak hooks instead; without a definition, Circle behaves exactly as upstream:

- ffsystem.cpp: ff_mutex_take / ff_mutex_give call OnyxFsLockTake (vol) / OnyxFsLockGive (vol) when they are defined, instead of the CGenericLock (a spin lock).

- emmc.cpp: once a TimeoutWait has spun 2 ms (the normal latency of a command; yielding earlier cost another task's whole time slice per command: reads fell to 2 MB/s), each turn of its loop calls OnyxDriverWait () when it is defined.

The kernel defines them in kernel/sys/fslock.cpp: a sleeping, re-entrant volume lock (waiters Yield), and a wait hook that yields when IRQs are on. See docs/02 (preemption).

7b (2026-10-01): many short waits. One command can now carry a whole file's contiguous clusters (patch 10): a long write waits for the card a few hundred µs per block, every wait under the 2 ms spin, and kept core 0 for 200 ms (stall: jet:cache ran 213 ms without yielding, the PC in TimeoutWait). emmc.cpp now also calls a second weak hook, OnyxDriverPoll (), at each turn of a wait shorter than 2 ms. The kernel's (fslock.cpp) yields once the task has run 10 ms since its last Yield (the scheduler notes that time), with the same conditions as OnyxDriverWait (a scheduler on this core, IRQs on, not core 1's crash dump). Safe for the same reasons as patch 7: the volume lock is held (no other task sends the card a command — and nothing outside FatFs drives the card while tasks run), and the SD host holds a PIO transfer between two blocks until it is served (upstream's NO_BUSY_WAIT yields at every turn of these waits). Reads (40 MB/s) yield about every 400 KB, writes every 50–100 KB. Without the hook, Circle behaves as upstream.

## 8. Sector cache in the FatFs disk layer

FatFs reads the FAT and the directories one sector at a time (through its window), and each disk_read is a whole SD command round trip (plus a CMD13 status check in the EMMC driver): opening a file deep in a big folder, or listing it, asked the card again for every sector, every time. diskio.cpp now keeps the single-sector reads in a direct-mapped cache (2048 slots x 512 bytes = 1 MB, allocated on first use; key = sector and drive). Write-through: disk_write writes the card first, then updates the cached copies of the sectors written (or drops them if the write failed), so the cache never holds anything the card does not. The big multi-sector reads of file data go straight to the device. disk_initialize (a mount) and the device's removal forget the drive's sectors. disk_cache_enable (0/1) / disk_cache_stats (&hits, &misses) (declared in diskio.h); Onyx turns it off with sdcache=0 in cmdline.txt.

Also: the bounce buffer for unaligned transfers is now one per volume (the SD driver can yield in the middle of a transfer since patch 7, so two volumes' transfers can overlap; FatFs locks per volume).

Test on the PC: tools/tests/run_fs_test.sh builds the fork's ff.c + diskio.cpp against stub Circle headers, formats a 96 MB RAM disk (FAT32) and runs 4000 random operations (create, overwrite, append, read back, delete, rename, list, remount) checked against a model, with the cache on and off: both pass and leave the same image byte for byte; the cache cuts the device reads about 3.4 times.

## 9. High Speed SD at run time

Upstream enables the SD High Speed mode (50 MHz, SD 1.1+ cards: CMD6 switch) only with the compile option SD_HIGH_SPEED, off by default, and then only raises the clock. The patch:

- static void CEMMCDevice::SetHighSpeed (boolean) (before Initialize) chooses it at run time (SD_HIGH_SPEED still forces the upstream behaviour); IsHighSpeed () tells whether the card switched;

- before raising the clock, it sets the host's High Speed Enable bit (CONTROL0 bit 2, HCTL_HS_EN of the SD Host Controller spec), so the controller samples with the High Speed timing;

- the 64-byte CMD6 status buffer is word aligned (the PIO transfer asserts it).

Onyx turns it on by default (sdhs=0 in cmdline.txt turns it off, for a card that misbehaves).

## 10. Multi-cluster transfers in FatFs

f_read / f_write move whole sectors straight between the caller's buffer and the disk, but upstream clips each transfer at the cluster boundary: with small clusters (a card formatted with 512-byte clusters) every SD command carried 512 bytes, and the commands' latency (~240 µs each on the Pi 4) capped reads at 2 MB/s (fsbench: 1 MB = 2064 commands; the data port itself moved 1 MB in 23 ms). The patch goes on across the next clusters while they follow on the disk (get_fat (cl) == cl + 1 for a read; create_chain — follow or allocate — for a write), up to the size asked, then sets fp->clust to the cluster of the last sector moved, so the loop resumes exactly as before. A cluster allocated ahead that does not follow simply stays in the chain (the next turn finds it, as it would have allocated it). Not with FastSeek (cltbl). The allocation order is unchanged: run_fs_test.sh gets the same disk image as upstream's ff.c, byte for byte, with 14 x fewer reads and 6 x fewer writes on 512-byte clusters.

## 11. FatFs fast seek

FF_USE_FASTSEEK is 1: f_lseek can use a file's cluster map (FIL.cltbl) instead of walking the FAT chain from the start at each backward seek. The kernel's kapi_seek (ABI v57) builds it at the first seek of a file bigger than 4 MB (f_lseek (fp, CREATE_LINKMAP), the table grown once if the file is fragmented) and kapi_close frees it: a GameCube disc image (1.4 GB) is then read anywhere at once. The option adds cltbl to FIL: the kernel must be rebuilt clean with the new libfatfs.

## 12. 2D DMA with a source stride, asynchronous partial updates

Why. Circle's SetupMemCopy2D only knows a destination stride: the source must be one contiguous block. So patch 6 gathered each damaged rectangle's rows into a buffer with the CPU (a copy as big as the DMA's) before the frame buffer's DMA copied it to the screen, and SetArea then busy-waited for the DMA: core 0 (the GUI, the apps' tasks) stood still for the whole transfer (2–4 ms for a full 1080p screen).

What. Both DMA engines have a source stride too (legacy STRIDE bits 0–15, DMA4 SRCI bits 16–31):

void SetupMemCopy2D (void *pDestination, const void *pSource,
                     size_t nBlockLength, unsigned nBlockCount, size_t nBlockStride,
                     unsigned nBurstLength, size_t nSourceStride);  // CDMAChannel, CDMA4Channel
void CBcmFrameBuffer::SetAreaPitch (const TArea &, const void *pPixels, unsigned nSourcePitch,
                                    TAreaCompletionRoutine *pRoutine = nullptr, void *pParam = nullptr);
void C2DGraphics::UpdateDisplayAsync (unsigned x, unsigned y, unsigned w, unsigned h,   // w = 0: all
                                      CDisplay::TAreaCompletionRoutine *pRoutine, void *pParam);

The overloads clean only the rectangle's rows from the data cache (not the whole span). The partial UpdateDisplay (x, y, w, h) of patch 6 now uses SetAreaPitch (the gathering buffer is gone). UpdateDisplayAsync starts the DMA and returns; the routine runs from the DMA's interrupt.

Polled, without the interrupt. Under a heavy GPU load (Ocarina of Time in n64emu) a completion interrupt was now and then never seen, and the compositor waited for ever (the screen, the apps presenting, the sound feeder behind them: all stuck). So the compositor (CCompositorTask::Present, kernel.cpp) no longer uses UpdateDisplayAsync; it starts the DMA and polls it between yields:

boolean C2DGraphics::UpdateDisplayStart (unsigned x, unsigned y, unsigned w, unsigned h); // FALSE: done at once
boolean C2DGraphics::UpdateDisplayPoll (void);                  // TRUE once the transfer is over
boolean CBcmFrameBuffer::SetAreaPitchStart (const TArea &, const void *, unsigned nSourcePitch);
boolean CBcmFrameBuffer::SetAreaPoll (void);                    // (frees the frame buffer's DMA channel)
boolean CDMAChannel::Poll (void);  boolean CDMA4Channel::Poll (void);   // not active any more

It draws nothing into the off-screen buffer until the poll says done. cmdline.txt dispdma=0 still makes every copy synchronous (UpdateDisplay).

## 13. SD card partitions as volumes, exFAT

Why. Upstream maps each FatFs volume to a whole device (emmc1, umsd1…): only the first FAT partition of the card was seen, and exFAT was off (files ≤ 4 GB, no exFAT user partition). The Pi 4 boots only from a FAT32 partition 1; a second partition, formatted exFAT, can hold the big files (ROMs, disc images).

What.

- ffconf.h: FF_VOLUMES 9, FF_VOLUME_STRS "SD","SD1","SD2","SD3","USB","USB2","USB3","FD","NVME" (since §28: 21 volumes, the USB devices' USB1..USB3 and their partitions USB1P1..USB3P4); FF_FS_EXFAT 1 (needs FF_USE_LFN; FSIZE_t becomes 64-bit → rebuild the kernel clean).

- ffconf.h: FF_MULTI_PARTITION 1. diskio.cpp: the physical drives are emmc1 (the whole card, as upstream), umsd1…umsd3, ufd1, nvme1; VolToPart maps SD: to the card's partition 0 (auto: FatFs takes the first FAT volume, exactly as before the change) and SD1:…SD3: to MBR partitions 2–4 of the same drive (their type is not checked: FAT or exFAT is recognised from the boot sector). disk_initialize keeps a drive already initialised (a second partition of it) instead of forgetting its sector cache. (A first version opened Circle's partition devices emmc1-1…: dropped, SD: must be found exactly as before.)

- ff.c: the multi-cluster write extension (patch 10) is skipped on exFAT: a contiguous exFAT file (NoFatChain) that becomes fragmented must first have its FAT chain written, which upstream does cluster by cluster (reads keep the fast path).

Rebuild everything that includes ff.h after changing ffconf.h: FF_FS_EXFAT makes FSIZE_t 64-bit, so FIL / FATFS / FILINFO grow. Besides libfatfs and the kernel (clean), that is addon/wlan (libwlan.a: the Wi-Fi firmware is read through FatFs) and addon/wlan/hostap/wpa_supplicant (make -f Makefile.circle: it reads wpa_supplicant.conf). With stale ones, their FIL on the stack is too small: FatFs writes past it on the network core — Onyx booted but the Wi-Fi never came up.

The kernel mounts SD1:…SD3: when f_mount succeeds and shares one lock slot between the four SD volumes (one card). tools/tests/run_fs_test.sh runs the FatFs test on FAT32 and exFAT images (the fork's image must equal upstream's), and first fstest parts: an MBR card image with partition 1 FAT32 and partition 2 exFAT, SD: and SD1: mounted, a file on each, SD2: absent.

## 14. Wi-Fi: a DHCP restart (join another network while running)

Why. CWPASupplicant reads wpa_supplicant.conf once, at boot, and once bound, Circle's DHCP client sleeps until its lease's renewal time: joined to another network, the Pi would keep the old address.

What. CDHCPClient::Restart () (static flag s_bRestart, include/circle/net/dhcpclient.h, lib/net/dhcpclient.cpp): the bound state checks it every 0.5 s (it slept 10 s at a time) and, if set, halts the network and starts over (discover / request).

The configuration itself is read again without touching hostap (a submodule of upstream's): wpa_supplicant registers its SIGHUP handler (wpa_supplicant_reconfig: every interface reloads its configuration, deauthenticates and joins the highest priority network in range) with eloop_register_signal_reconfig, which Circle's eloop ignores. The Onyx kernel is linked with --wrap=eloop_register_signal_reconfig (kernel/Makefile): it keeps that handler, and kapi_wlan_reconnect (ABI v60) runs it from the supplicant's own event loop (a 0 s eloop timeout), then asks CDHCPClient::Restart ().

## Not a patch: build configuration

One required setting is a configure option, not a source change: Onyx renders 32-bit pixels, so Circle must be configured with DEPTH=32:

cd circle && ./configure -r 4 -p aarch64-none-elf- -d DEPTH=32 -f

See the Developer Guide §2 for the full Circle build.

## 15. A crash area kept out of the heap

Why. A frozen Pi (a kernel hang, a bus wedged by the GPU) leaves nothing to read: kmsg is gone with the session. Onyx keeps a crash record in RAM that survives the watchdog's reboot.

What. CMemorySystem::SetupHighMemAbove4G (lib/memory64.cpp) keeps the top ONYX_CRASH_AREA_SIZE (64 KB) of the first RAM segment it adds above 3 GB (the [3 GB, RAM top) low-RAM top from the device tree, or the [4 GB, RAM end) chunk of the fallback — the case of an 8 GB Pi 4 without a captured device tree) out of the high heap and publishes its address in u64 g_ulOnyxCrashArea (include/circle/memory.h; 0 when the board has no RAM there, e.g. 2 GB).

CSpinLock::TryAcquire () (include/circle/spinlock.h, lib/spinlock.cpp): one try, FALSE (nothing held, the IRQ level restored) when another core holds the lock — core 1's crash watch never waits for ever behind core 0. It stays mapped NORMAL (cacheable): the kernel cleans each write to the point of coherency. See docs/02 §13.

## 16. TCP: a peer gone before it was accepted

Why. A client that connects and closes at once (a port scan, a script checking a port) left the listening connection woken without a foreign address: CTCPConnection::Accept then called CIPAddress::Set on it, whose assertion (rAddress.m_bValid) is a panic — every core halted, the Pi restarted (seen with telnetd: lastcrash.txt showed it, in a restart loop while a monitor on the PC probed port 23).

What. lib/net/tcpconnection.cpp, Accept: after the wait, an error (m_nErrno) or a connection without a foreign address returns -NET_ERROR_CONNECTION_RESET; CSocket::Accept then returns 0 (that peer not accepted) and listens again. The same bug is in upstream Circle.

## 17. The host name set after the constructor

Why. The name DHCP announces is given to CNetSubSystem's constructor, and Onyx builds it as a member of CKernel — before the SD card is mounted, so before SD:/etc/system.ini (its hostname=, written by Setup, the first-run wizard) can be read.

What. include/circle/net/netsubsystem.h: void SetHostname (const char *pHostname) (inline: m_Hostname replaced), called by the kernel before the network's bring-up task starts (Initialize makes the DHCP client with the name). Header only: no library to rebuild.

## 18. DNS: the answer polled, not a second slept

Why. CDNSClient::Resolve sent its query, slept 1000 ms, then read the answer once: every name lookup cost a second even when the answer came in 5 ms (a web page asks for several hosts: NetSurf spent seconds in DNS alone, its connects one after another).

What. lib/net/dnsclient.cpp: after each send, the socket is read every 5 ms (MSG_DONTWAIT) for up to 1000 ms; the three tries are kept. (The kernel also caches the answers: kernel/sys/net.cpp, docs/02.)

## 19. TCP: a 64 KB receive window

Why. The advertised receive window was 10 segments (14600 bytes), with no window scaling: a connection could not carry more than 14.6 KB a round trip (about 730 KB/s at 20 ms, 290 KB/s at 50 ms) -- well under the Wi-Fi's rate.

What. lib/net/tcpconnection.cpp: TCP_CONFIG_WINDOW is 44 segments (64240 bytes, the most a 16-bit window field holds); the reassembly queue follows it. Rebuild lib/net.

## 20. TCP: real duplicate ACKs only

Why. CTCPConnection::PacketReceived counted every segment whose ACK did not advance as a duplicate ACK — the peer's own data segments too, and ACKs with nothing in flight, window updates — and the count was reset only by an advancing ACK. The remote desktop client (Onyx Remote) sends small messages (READY, the pointer's moves) while the Pi streams the screen: three in a row between two ACKs started a spurious fast retransmit + fast recovery (cwnd cut to about two segments), and the Pi's sending crawled while the mouse moved.

What. lib/net/tcpconnection.cpp: in the "ACK not advancing" branch, OnDuplicateAck is called only for a duplicate ACK as RFC 5681 §2 defines it: no data and no SYN / FIN (nSEG_LEN == 0), the ACK number SND.UNA, the advertised window unchanged (m_nSND_WND), and data outstanding (FLIGHT_SIZE > 0). Other segments neither count nor reset the count. Fast retransmit and fast recovery (NewReno) are unchanged otherwise. The same bug is in upstream.

## 21. TCP: the retransmission timeout

Why. retranstimeoutcalc.cpp started at 3 s (RFC 1122's value) and never went below 1 s (RFC 6298's conservative minimum): on a LAN or Wi-Fi peer a few ms away, a segment lost at the end of a burst (nothing behind it to cause duplicate ACKs) stalled the stream a whole second — the remote desktop froze.

What.

- INITIAL_RTO 1 s (RFC 6298 2.1); MIN_RTO stays 1 s. A 200 ms minimum (Linux's TCP_RTO_MIN) was tried on 2026-10-01 and undone the same day: the remote desktop's throughput over Wi-Fi to a Windows PC collapsed (64 KB sends taking 0.5–2 s instead of a few ms, ~50x slower). Windows delays its ACKs up to 200 ms and Wi-Fi adds bursts of delay: the timer fired spuriously and each timeout cut the congestion window to one segment; Linux survives that floor with F-RTO / undo of spurious timeouts, which Circle lacks. RFC 6298's granularity term G is 200 ms (was 1 tick).

- Karn's algorithm also after a fast retransmit: CRetransmissionTimeoutCalculator::SegmentResent (new) marks a segment sent again by ResendSegment (fast retransmit, a partial ACK in fast recovery), so its ACK gives no RTT sample (only a timeout marked it before). A valid sample resets the backed-off RTO, as before.

- MAX_RETRANSMISSIONS stays 5 (63 s from 1 s), MAX_SYN_RETRANSMISSIONS 5 (63 s from the 1 s initial RTO; it was 189 s from 3 s).

- After a SYN retransmitted, RFC 6298 (5.7) wants the data phase to start at 3 s: the RTO is then the backed-off one (≥ 2 s) until the first sample — close enough, left as is.

## 22. CSocket::Send: the bytes queued

Why. CSocket::Send cuts a buffer into MSS-sized chunks and queues them one by one; when the connection's queue stays full past the send timeout (Onyx sets 5 s), a chunk fails — and Send answered the error, although the chunks before it were queued and would be sent. The kernel's NetTcpSend (32 KB requests) then counted none of that request, and an app sending "the rest" again put those bytes twice in the stream (the remote desktop's stream broke: its client read garbage).

What. lib/net/socket.cpp: Send (and SendTo, for TCP) answers the bytes queued when a later chunk fails, if any (as POSIX send); the error comes back at the next call. The kernel's NetTcpSend adds its requests' counts and stops at a short one.

Test (patches 20–22): tools/tests/run_circlenet_test.sh builds the fork's real tcpconnection.cpp, retranstimeoutcalc.cpp, socket.cpp (+ the buffers, queues, checksum) on the PC against stub Circle headers (tools/tests/circlenet/stub: a simulated clock and kernel timers, no tasks, a network layer that hands each segment to the test) and plays the peer: the peer's data segments and window updates are no duplicate ACKs, three real ones still start a fast retransmit and a full ACK ends the recovery, the RTO (200 ms, backed off, reset by a sample, 1 s for a SYN, Karn after a fast retransmit, a dead peer given up after 51 s), and Send's count after a timeout. Against the unpatched sources it fails 15 of its 25 checks.

## 23. Sockets for the BSD layer: AcceptReady, a connection kept until its socket lets it go

Why. The kapi v75 BSD sockets (docs/02 §8, sys/bsdsock.cpp) need a non-blocking accept and sockets that stay valid after a reset until close. Upstream deleted a terminated connection at the next Process even while a CSocket still held its handle; after a reset the handle went to the next connection, which the old socket then read, wrote and closed.

What (fork commit f1d6b200; tools/circle-patches/wp-net.patch): CSocket::AcceptReady () (a backlog connection already connected, so Accept will not block; it also replaces backlog connections that died before being accepted); CNetConnection::SetReleased / IsReleased, set by CTransportLayer::Disconnect and by a failed Connect — a terminated connection is deleted only once released; CSocket::Accept disconnects a failed backlog connection instead of leaking it; CTransportLayer::IsTerminated (h).

Test: tools/tests/run_circlenet_test.sh (the stub gained IsTerminated): 25/25.

## 24. CPageAllocator::Allocate: a failed attempt leaves the count as it was

Why. Allocate advanced its bump pointer (m_pNext += PAGE_SIZE) before checking it against m_pLimit, and returned 0 without stepping it back: every failed attempt left the pointer one page further past the limit, and GetFreeSpace () (m_pLimit - m_pNext, a size_t) wrapped. palloc_high tries the high segments in order, so once segment 0 was full every page served elsewhere cost it one more page: meminfo's free memory fell for ever (an OOM kill seemed to lose ~1.1 GB on an 8 GB Pi) and the kernel's OOM check fired with half the memory still free. No frame was lost.

What. lib/pageallocator.cpp: m_pNext -= PAGE_SIZE before that return 0.

Test: memtest oom on the Pi (the OOM kill twice, each run ending where it started).

## 25. TCP: a closed connection's retransmission timer

Why. The Pi restarted by itself under network load (a video streamed by the WebKit browser, an FTP upload of 100 MB and the remote desktop at once; a telnet client gone while output flowed was enough): SD:/etc/lastcrash.txt said A kernel panic: tcp: Unexpected state 0 at line 1922. That line is CTCPConnection::TimerHandler's retransmission case: the timer of a connection whose state is Closed. A peer's RST, the ACK of our FIN in LAST-ACK or a refused connect closed the connection with the retransmission timer still running; since §23 a terminated connection is kept until its socket releases it (upstream deleted it at the next Process), so the timer fired on the closed connection a second later. Upstream calls that UNEXPECTED_STATE (), which in a build without NDEBUG — ours — is LogPanic: every core halted, for one connection, and any client could cause it.

What. lib/net/tcpconnection.cpp:

- every path of PacketReceived that goes to Closed (a RST in each state, a SYN in the window, the FIN's acknowledgement in LAST-ACK, a refused connect) stops the retransmission timer first;

- TimerHandler, TCPTimerRetransmission: in Closed, Listen, FinWait2 or TimeWait the timer is ignored before the RTO is backed off or the retry count taken (the handler runs from an interrupt, with netcore=1 on another core than the stack: it can still race those paths);

- a reset (and the SYN case) also sets m_TxEvent: a sender waiting for room in the transmit queue sees the reset at once instead of at its timeout.

Test: on the Pi — YouTube playing in Web with an FTP upload of 100 MB and telnet sessions opened and dropped: no panic since (it came within minutes before).

## 26. Wi-Fi: the chip polled, a scan over both bands, 5 GHz preferred

Three changes in the BCM4343x driver (addon/wlan/ether4330.c), each behind a global the kernel sets (kernel/sys/net.cpp: NetWlanOptions, NetWlanNames); with the globals at their defaults the polling and the bias are off (the scan over both bands is not conditional). hostap — upstream's submodule, wpa_supplicant and its Circle driver glue — is not changed.

1. The chip polled (onyx_wlpoll). The receive loop (rproc) read frames until the chip had none, then slept in intwait until the SDIO card interrupt. That interrupt comes late: measured on a Pi 4 under a steady stream, 2.5 to 5 ms a wait, 80 % of the reader's time, with the frames already there — the Pi sent 1.1 MB/s and received 1 MB/s on a link good for forty times that. With onyx_wlpoll set (the kernel does, when the network has a core of its own: netcore=1), the reader asks the chip itself, in turn with the stack's other tasks — a round of that core's scheduler is ~13 µs: intpoll reads the chip's interrupt status (function 0's Intpend, then the backplane's Intstatus, acknowledged) and tells whether FrameInt is pending; the data function is read only then, and until it is empty (as Linux's brcmfmac: no read with nothing pending). On the primary core (netcore=0) the driver waits for the interrupt as before: polling would take the core from everything else.

2. A scan over both bands, the networks probed by name. wlscanstart listed the fourteen 2.4 GHz channels with the wildcard SSID alone, so a 5 GHz BSS was never a candidate: a Pi 4 beside a dual-band access point joined it on 2.4 GHz channel 1, shared with the neighbours. The escan now passes nchans = 0 (the firmware's own list: every channel of both bands the country allows) and, beside the wildcard, up to four names (onyx_scan_ssid, onyx_scan_nssid): an access point that leaves its name out of the beacons of one band (seen on a Livebox: the 5 GHz BSS came with an empty SSID) answers a probe by name and is seen as that network. The kernel fills the names from the ssid="…" lines of SD:/etc/wpa_supplicant.conf before wpa_supplicant starts and at each wlan_reconnect. A scan takes longer (5 GHz's DFS channels are listened to, not probed): wlan_scan waits 5 s instead of 3.5.

3. 5 GHz preferred (onyx_scan_5g_bias). wpa_supplicant ranks the BSSs of a network by level, and 2.4 GHz is usually heard a few dB louder. wlscanresult adds the bias (the kernel: 25 dB) to the level of each 5 GHz BSS heard at −78 dBm or better before the result is queued (kept negative), so a network on both bands is joined on 5 GHz unless that band is much weaker. The kernel's own scan (wlan_scan, wifiscan) clears the bias for its length: its list shows the levels as heard. (The glue reads the level unsigned — res->level = bss->RSSI, a u16: −67 is 65469 — an upstream bug left as it is: the order stays the same.) WLC_SET_ASSOC_PREFER was tried first and changes nothing here: the supplicant names the BSSID it joins.

addon/wlan/p9util.cpp gains p9usec (the microsecond clock) and p9yield (a turn of the scheduler) for the driver's C code. With onyx_wlstat (the kernel: cmdline.txt netstat=1) the driver logs every 5 s under load its frames a second, a frame's read and write times, the waits, and every 10 s what the firmware says of the link (rate, RSSI, chanspec, power save).

4. The frames' path (onyx_wlfast, a bit each; the kernel sets 31). With the chip polled and 5 GHz joined, a frame's read still took ~270 µs — 3700 frames a second at best:

- bit 1 — one command a transfer (packetrw): sdiorwext sent a length over a block (512) as its whole blocks in one CMD53 and the rest in a second, byte-mode one: a 1500-byte frame was the header (12 bytes), two blocks, then 464 bytes — three commands. A length over a block is now rounded up to whole blocks (the "roundup" of Linux's brcmfmac: past a frame's end the chip pads a read and ignores what is written).

- bit 2 — the next frame read whole (wlreadpkt): a frame's header tells the next frame's length (nextlen, 16-byte units) when the chip has one queued; that frame is then read in one command instead of its header first. A hint too short is completed by a second read.

- bit 4 — the frames' locks without a yield (fqlock / fqunlock, emmcio): qlock and qunlock each call the scheduler, free or not, and tsleep yields before it looks at its condition — a frame cost eleven turns of the scheduler (the packet lock, the SDIO lock around each command, the wait for each transfer's end), each a round of all the stack's tasks. On the frames' path a free lock is taken and released at once, a transfer already ended is not waited for, and the reader yields once a frame. (Doing this in p9proc.cpp for every lock and sleep of the driver cut the Wi-Fi at boot — not found why; only this path is changed.)

- bit 8 — the bus at 50 MHz (emmc.c): the driver switches the card to High Speed (function 0's register 0x13) but the host stayed at 25 MHz; when the bus goes to four lines the host now takes High Speed timing and 50 MHz, as Linux runs this chip on a Pi 4. (The controller's base clock is 250 MHz there: the divider gives 41.7 MHz. The next step, 62.5 MHz, was tried: the chip does not answer.)

- bit 16 — the controller's registers written without a wait (emmc.c, WR): each write waited 2 µs first — two periods of the SD clock, which is 2 µs at 1 MHz and 40 ns at 50 —, and a command writes eight registers.

A frame's read: 93 µs (was 270), a frame's write 16 µs (was 36).

Receive glomming was tried and taken out. bus:rxglom is accepted by the firmware, which then expects the glom header on what it is sent too (brcmfmac's txglom: 8 bytes between the length words and the software header — without it the chip stops answering commands); with both done the Wi-Fi works, but this firmware (the 43455's) never sent a superframe in 50 MB received, and the rates were a little lower.

Measured (a Pi 4, tcpbench against a PC on the same access point; with the kernel's inter-core interrupt of docs/02 §11, which came first, and §27's TCP changes, which came last):

|  | echo round trip | the Pi sends | the Pi receives |
| --- | --- | --- | --- |
| before | 10.4 ms | 795 KB/s | 504 KB/s |
| the IPI (kernel) | 2.8 ms | 1128 KB/s | 1008 KB/s |
| + the chip polled | 2.8 ms | 3.3 MB/s | 1 MB/s |
| + 5 GHz (VHT, 80 MHz, link 195–292 Mbit/s) | 2.2 ms | 4.3 MB/s | 4.4 MB/s |
| + TCP window scaling (§27) | 2.3 ms | 6.6 MB/s | 4.6 MB/s |
| + one command a frame, the locks | 2.3 ms | 6.4 MB/s | 5.5 MB/s |
| + the bus at 50 MHz | 2.2 ms | 7.5–8.1 MB/s | 6–7 MB/s |
| + the registers written without a wait | 2.1 ms | 8–9.5 MB/s | 8.7 MB/s |

From the internet (the Pi's curl): 30 MB from Cloudflare at 7.1–8.5 MB/s (it was 0.8, then 2.2–2.8 with the first four rows); YouTube's 3 MB player script in 0.6 s (was 4 to 23 s). No error of the driver in 4 × 64 MB each way.

What limits it now (netstat=1): 6000 frames a second at 93 µs each is over half the reader's time — 61 µs of it is the frame's data on the bus —, and the Pi acknowledges every segment (6000 frames sent a second while it receives). What is left: delayed acknowledgements. The bus is on four lines (Busifc 2) and the link is VHT (vhtmode 1, chanspec e02a): neither is the limit.

5. The frames sent are not aggregated (onyx_wl_ampdu_tx = 0: the firmware's ampdu_tx). The symptom was the user's: while the browser loads a page, the remote desktop freezes. Measured: during a download from the internet the PC's pings of the Pi lost 20 % of their answers, and 50 to 75 % during a download limited to 1 MB/s (none when idle); an echo on another TCP connection came back after 1, 3, 7 seconds (the Pi's retransmission timeouts: its minimum is 1 s), a telnet session stalled for a minute. Where the packets went, step by step:

- the Pi's network core is never held (its longest scheduler round in 5 s: 0.65 ms);

- the Pi receives every request and writes every answer to the chip (the driver's counts of ICMP frames handed up and taken down: 150 / 150), never keeping a frame more than 19 ms;

- the PC receives a third of them (Windows' counters: no damaged packet, just fewer), and the Pi's own pings to the gateway are lost alike at that time (8 answered of 25): it is what the Pi sends that disappears, whatever its destination — its TCP acknowledgements too, which a download does not notice (they are cumulative);

- the firmware counts them as sent and acknowledged (its counters: txfail 2 in 3453 frames).

So the frames are lost after the access point's block acknowledgement — in the firmware's or the access point's handling of the aggregates (an Orange Livebox, 5 GHz, VHT 80 MHz, −70 dBm). What changes it: ampdu_tx 0 — none lost (wlinit: the interface taken down, the setting, up again; it is refused while up). What does not: a 40 MHz channel (bw_cap), a block acknowledgement window of 8 (ampdu_ba_wsize), no RTS (ampdu_rts), no A-MSDU, a receive window of 8. The price is the Pi's own sending rate — each frame is a transmission of its own — and the acknowledgements of what it receives: hence §27's delayed acknowledgements.

| the kernel's defaults now | the Pi receives | the Pi sends | from the internet | the PC's pings lost during a download | an echo during a download |
| --- | --- | --- | --- | --- | --- |
| A-MPDU on what is sent (the table above's last row) | 8.7 MB/s | 8–9.5 MB/s | 7–9 MB/s | 20–75 % | up to 7 s |
| no A-MPDU on what is sent, an acknowledgement for 8 segments | 3.8–4.8 MB/s | 1.3–1.7 MB/s | 5.6–5.9 MB/s | 0 % | 77 ms at worst |
| + frame bursting (onyx_wl_frameburst = 1: the firmware's command 219, which Linux's brcmfmac sets too — several frames in one transmit opportunity) | 4 MB/s | 3.1 MB/s | 6.5 MB/s | 0 % | 0.3 s at worst |

The settings are the driver's globals (onyx_wl_ampdu_tx, _ampdu_rx, _ba_wsize, _ampdu_rts, _rx_ba_wsize, _bw5, _frameburst; −1: the firmware's own), each a word of the trial file. To do: whether a newer firmware aggregates soundly (the sending rate would be 6 to 9 MB/s again).

Trying such a change on a Pi that is only reachable by its Wi-Fi: the kernel's one-boot trial file (docs/02 §11 A trial): each of the bits above was tried alone that way before it became the default.

### The driver tells when the chip was last busy

ether4330.c keeps two figures for the kernel's network core, which sleeps between two questions to the chip once the network is quiet (docs/02, The network core sleeps when the network is quiet): onyx_wl_lastact, the clock when the chip last said it had something, when a frame was last read from it or written to it; onyx_wl_polls, the times the polling reader asked the chip and it had nothing. No change of behaviour in the driver itself.

emmc.c: sdiocardintrpending (arm) tells whether the card's interrupt is pending, from the controller's flag (no command) and, when it is not and arm is set, enables that interrupt — its handler then sends an event (sev) that ends the network core's sleep. Two things had to be put right for it. The flag is the line's level only while its status is enabled: written alone it stays set after the data lines moved during the last commands (it read "pending" for ever) — its status is disabled, then enabled again, and it is sampled anew, as Linux's SDHCI driver does. And the interrupt enable register is changed from two cores, the driver's tasks setting bits and the handler clearing those that came: every such change is now made under one lock (irpenable, mmcinterrupt). sdiodebugreg gives the controller's registers to the kernel's statistics line.

## 27. TCP: window scaling, a receive window that follows the queue

Why. The receive window was a constant — 64240 bytes (§19), never scaled, never smaller: (1) a connection carried at most 64 KB a round trip — 2.5 MB/s at 25 ms from the internet, and 4.5 MB/s on the LAN, where the queueing makes the round trip 14 ms; the Pi's own sends were held to 64 KB too (the peer's window read without its scale, a 64 KB transmit threshold); (2) a reader that stopped reading did not stop the peer: the receive queue grew without a limit (a paused download, a video's loader that waits).

What. lib/net/tcpconnection.cpp (the switch onyx_tcp_ws, set by the kernel; 0: as before):

- the option: our SYN carries window scale 3 (after the MSS: NOP, kind 3, length 3); a SYN+ACK carries it when the peer's SYN did. ScanOptions takes the peer's shift (in a SYN, in SYN-SENT or LISTEN); with both, the peer's window field is shifted left by its shift (never in a SYN), ours right by 3, and the receive limit is 180 segments (262800 bytes). TCP_MSS_HEADER_LEN is 28 (the buffer's headroom).

- the window sent is the room left in the receive queue (ReceiveWindow: the limit less the bytes queued), in every segment; Receive — the reader made room — sends a window update once that opens the window by a quarter of the limit over what the peer was last told.

- the transmit threshold (when Send waits, when a socket is writable) is 256 KB, the initial slow-start threshold 1 MB with a scaled peer (it was 65535: linear growth past 64 KB).

- ScanOptions stops at an option of length under 2 (it looped for ever).

- delayed acknowledgements (onyx_tcp_ws bit 2, onyx_tcp_ackn: the kernel sets 8): every data segment was acknowledged by a segment of its own — 6000 frames a second sent while receiving 9 MB/s, each a transmission on the radio once they are no longer aggregated (§26, 5). A full segment (1000 bytes or more) in order, without PUSH or FIN, that fills no hole, is counted; the acknowledgement goes with the eighth, or from Process 10 to 20 ms after the first (m_nAckPending, m_nAckPendingTicks); a short segment (a request, a keystroke), a PUSH, an out-of-order segment are acknowledged at once, as before. Any segment sent with ACK clears the count.

- (onyx_tcp_trace, the trial's netstat=1: a line at each retransmission timeout — the connection, its sequence state, how long ago the peer's last segment came.)

Measured: the table of §26 (the Pi sends 4.3 → 6.6 MB/s by this alone); the download from Cloudflare 2.2–2.8 → 6.0 MB/s with §26's fast path.

## 28. USB volumes: format, labels, an unmount while a call waits, the eject

Why. kapi v93 (docs/02 §18) mounts the USB mass-storage devices as USB1:, USB2:, USB3: (or USB1P1:… per partition) when they are plugged in and unmounts them when they are ejected or pulled out, formats a volume and shows its label. Upstream's FatFs configuration has neither f_mkfs nor the labels, and FatFs assumes a volume is not unmounted while a call on it waits for its lock.

What.

- ffconf.h: FF_USE_MKFS 1, FF_USE_LABEL 1 (f_mkfs, f_getlabel, f_setlabel). Neither changes a structure: no clean rebuild, libwlan / wpa_supplicant unaffected (unlike §13).

- ff.c: an unmount while another task waits for the volume lock (a USB stick ejected or pulled out: the kernel unmounts it holding the lock, the waiter goes on once it is given). validate tests the object again once the lock is held (fs_type, id; before, only the drive's status): a write on a volume unmounted meanwhile was carried out on the stale FATFS — on an ejected stick still plugged in, written to it behind the user's back. mount_volume checks FatFs[vol] is still the object it locked: it was re-mounting the unregistered object (an open after the eject found the volume back by itself). Both now return FR_INVALID_OBJECT / FR_NOT_ENABLED and give the lock back.

- ffsystem.cpp: with the kernel's lock hooks (§7) the CGenericLock is never used; ff_mutex_delete (at an unmount) no longer deletes it and ff_mutex_take / _give call the hooks before asserting it exists: a task that took the lock before the unmount gives it back after, which asserted (a kernel panic) before. ff_mutex_create keeps the existing one (a remount). Without the hooks: upstream's behaviour.

- The USB devices' partitions (the user's naming, 2026-10-06): ffconf.h FF_VOLUMES 21, the strings "SD","SD1","SD2","SD3", "USB1","USB1P1".."USB1P4", "USB2",.., "USB3",.."USB3P4", "FD","NVME"; diskio.cpp's VolToPart: USBn = {n, 0} (the device whole: its first FAT volume, a superfloppy or its only partition), USBnPm = {n, m} (MBR partition m). The kernel mounts one or the other by the device's sector 0 (docs/02 §18). ff.c: its check FF_VOLUMES <= 10 lifted to 32 — it only guards the numeric 0:..9: form (get_ldnumber reads one digit), which Onyx does not use; the volumes are reached by name. The structures do not change (no clean rebuild; tools/tests/run_fs_test.sh lifts the same check in its copy of upstream's ff.c).

- diskio.cpp: after each transfer with a drive other than the SD card (pdrv != 0), the weak hook OnyxDriverPoll (the kernel's, §7b: it yields once the task has run 10 ms). The USB driver waits in a busy loop (NO_BUSY_WAIT off): a format, a big folder kept core 0 for seconds. Between two transfers is a safe point: the drive's lock is held (the kernel's fslock.cpp: one lock per physical drive, so one for all the volumes of a USB device), it is called once the bounce buffer and the sector cache are done with, and a device removed meanwhile only fails the next transfer (disk_removed cleared s_pVolume).

- usbmassdevice.{h,cpp}: IOCtl (DEVICE_IOCTL_SYNC) — FatFs' CTRL_SYNC — sends SCSI SYNCHRONIZE CACHE (10) (whole medium); a device that refuses it (no cache) is reset to a known state and the call succeeds. The kernel sends it at an eject, after a format and at the session's end.

Tested on the PC: tools/tests/run_fs_test.sh builds tools/tests/fs/usbtest.cpp with the fork's FatFs and FF_FS_REENTRANT 1 (a counting lock of its own): sticks formatted as FAT32 / exFAT / FAT16 with an MBR (as Windows), a FAT32 superfloppy, all found by USB1:'s auto search; a device of two partitions (USB1P1: FAT32, USB1P2: exFAT) then made one again by a format of USB1:, labels set and read; a stick pulled out while a file is written (errors, the lock free, the old objects invalid on the next stick); an unmount while a call waits for the lock — which fails with upstream's ff.c (the write accepted, the open succeeding). Not tested on the Pi yet (docs/HANDOFF.md).

## 29. The Pi 5's high RAM registered once

Why. On the Pi 5 (RASPPI = 5) MEM_HIGHMEM_END is 8 GB and the RAM is one block (the I/O is above 64 GB, no hole at 3-4 GB): SetupHighMem already makes seg0 = [1 GB, min (RAM, 8 GB)). Onyx's SetupHighMemAbove4G (§15 and the high-memory patch) then found nothing above 8 GB in the device tree and its fallback (B) registered [4 GB, 8 GB) a second time: two page allocators handing out the same frames on an 8 GB board.

What. lib/memory64.cpp, under #if RASPPI >= 5: SetupHighMem takes the crash record's 64 KB (ONYX_CRASH_AREA_SIZE, g_ulOnyxCrashArea) from seg0's top, and SetupHighMemAbove4G is not called. RAM above 8 GB (a 16 GB board) stays unused until the Pi 5's user window moves above it (docs/PI5-PORT.md §5.2, policy B). g_ulOnyxCrashArea is defined before SetupHighMem now. The Pi 4's code is unchanged (its object differs only by the assert line numbers). Fork commit 065871f9; built in circle5/ by tools/pi5/circle5.sh. Not tested on a Pi 5 yet.

## 30. The net device chosen at run time

Why. CNetSubSystem is made once, in CKernel's constructor, with its device type (Onyx: the WLAN). The Pi 5 has Gigabit Ethernet (the RP1's MACB, which Circle drives): Onyx takes the cable when its link comes up at boot, else the Wi-Fi (system.ini network = auto | ethernet | wlan, docs/04) -- a choice made after the stack is initialized, since the MACB is brought up by CNetSubSystem::Initialize.

What. CNetDeviceLayer::SetDeviceType (TNetDeviceType) (include/circle/net/netdevlayer.h, lib/net/netdevlayer.cpp): the type set and the device forgotten, so the next Process () looks it up again. Called before the link is up (CNetSubSystem::Process starts the DHCP client on the first device running), so no address was asked on the other one. The kernel (kernel/kernel.cpp, CNetBringupTask) calls it through GetNetDeviceLayer (). Unused with network = wlan (the Pi 4's default): the Pi 4 runs as before. Not tested on a board yet. Fork commit ec821a94.

## 31. The Pi 5's presented pixels made opaque

Why. The first test on a board (a Pi 500, 2026-10-09, a tester's report): the boot log showed, then the screen went black when the desktop came, the ACT LED still blinking (the kernel alive). Onyx's pixels are 0x00RRGGBB (the alpha byte 0) and the Pi 5's display takes the alpha: the log's text, Circle's colours with alpha 255, showed; the desktop, alpha 0, was transparent. framebuffer_ignore_alpha=1 in config.txt (on the Pi 5's card) was not honoured there.

What. C2DGraphics::Opaque (include/circle/2dgraphics.h, lib/2dgraphics.cpp, #if RASPPI >= 5 && DEPTH == 32): the rectangle's alpha bytes set to 0xFF in the off-screen buffer before it is sent (UpdateDisplay, the partial UpdateDisplay, UpdateDisplayAsync, UpdateDisplayStart) -- one pass over the presented rows, before the DMA cleans them from the cache. The Pi 4 is unchanged. With it, the kernel's direct mode (fullscreen_direct: a program writing the displayed framebuffer itself, n64emu and gcemu) is off on the Pi 5 (MapScreen gives 0: the programs present through fullscreen_begin / present_fb). Fork commit 9266885b. Switchable (2026-10-10): C2DGraphics::s_bOnyxOpaque (default TRUE), set from the kernel's cmdline.txt opaque= -- the tester found that the Pi 500's latest bootloader honours framebuffer_ignore_alpha=1 (Raspberry Pi OS also works around it in software); opaque=0 skips the pass and gives the direct mode back. The Display applet has the check box. Fork commit b47451c6. Tested on a Pi 500 (2026-10-10): the desktop shows.

## 32. The Japanese keyboards' Ro and Yen keys

Why. A Japanese (JIS) keyboard -- the Raspberry Pi 500's and keyboard's JP variant -- has two keys past the layout table's 128 rows: Ro (\ _, USB usage 0x87, International1) and Yen (¥ |, 0x89, International3); CKeyMap::Translate dropped them.

What. lib/input/keymap.cpp, CKeyMap::Translate: 0x87 is taken at row 0x7D and 0x89 at 0x7E (Paste and Find, which no layout maps), where SD:/etc/keymaps/JP.kmap has them. The raw keys (kapi_key_held, the games) are unchanged. Not the keymap decoupling of §1 (its skill re-applies that one after an upstream merge: check this fold is still there too). Fork commit 9a9180ba. Not tested on a Japanese keyboard yet.

## Contributions to upstream Circle

The fork's changes useful to every Circle user are prepared as clean pull-request branches on upstream develop (the scheduler's WakeTasks race, the SD High Speed fixes, large heap blocks reuse, partial display updates with a DMA source stride, the DHCP restart), with the patches and the pull-request texts: see docs/circle-upstream/README.md.

## Upstream merges

2026-10-09: Circle 51.1 and 51.1.1 (84 upstream commits after Step51, merge c06e5a52). What Onyx gains: the xHCI endpoint recovered from Halted after a transfer error (#704: the Pi 4's and the Pi 5's USB); transient HID report errors tolerated, a generic HID mouse-shaped report taken as a mouse (#693); the 8BitDo controllers, keyboards and Wireless Adapter 2, the Xbox 360 wireless receiver (and clones); Wi-Fi: the eero 7 routers (#696) and the hidden networks (hostap at circle-step51.1); FatFs R0.16 patch 2 (the Onyx patches merged automatically: tools/tests/run_fs_test.sh passes); the lost wake-up of CSynchronizationEvent (BlockTask checks the state under its lock) — ported into Onyx's own scheduler (kernel/sched/scheduler.cpp, kernel/compat/circle/sched/scheduler.h: it replaces Circle's); for the Pi 5: the official DSI touchscreens (addon/rp1dsi), the RP1's I2C buses 4 and 6. Upstream's DWHCI / usbboost / USB-MIDI work is for the Pi 1-3 only. Three conflicts: usbkeyboard.h (our GetKeyMap () beside upstream's protected ConfigureKeyboard), ether4330.c (our scan kept — both bands, the configured networks probed by name — with upstream's directed probe for a hidden network added as one more name), tcpconnection.cpp (our duplicate-ACK test kept: patch 20, the same RFC 5681 test without resetting the count on another segment). The keymap decoupling (patch 1) came through intact. Tested on the PC (run_fs_test.sh, run_circlenet_test.sh, run_ramfs_test.sh, run_gui_test.sh, run_gamepad_test.sh). Tested on the Pi 4 (the user, 2026-10-09): the Wi-Fi joins, TCP holds (YouTube audio and video in Jet, a remote desktop session over rdpd), the Game Boy Color emulator plays with its sound. Not checked yet: a keyboard unplugged and plugged again (its layout kept), the 8BitDo / Xbox 360 wireless devices.

## Updating the fork (submodule)

# merge a newer upstream into the Onyx branch, inside the submodule (a merge, not a rebase: the
# branch is published and the superproject pins its commits):
git -C circle fetch upstream --tags
git -C circle merge upstream/master            # resolve conflicts in the patched files
# then the skill resync-circle-keymap-patch (the keymap decoupling), and the Pi 5's tree: sh tools/pi5/circle5.sh
# rebuild the affected libraries: libcircle (heap + page accounting), libinput (keymap)
# then record the new fork commit in the superproject:
git add circle && git commit -m "Bump circle submodule"